COLLECTED DATA FOR MODEL

Relating to lightbulb / energy use

- Energy cost per kWh:
27.03 pence per kWh (Q2 2025 price cap, includes VAT)
Source: Energy price cap | Ofgem

- CO₂ emissions per kWh:
0.207074 kg CO₂e per kWh (2023 UK grid average)
Source: Greenhouse gas reporting: conversion factors 2023 - GOV.UK, condensed set, under “UK electricity”

- Hours a day a lightbulb is used:
3.9 hours a day (Table 5, calculated by taking mean of medium)

Source: Energy Follow-Up Survey 2011 Report 8: Lighting

- Average lightbulbs per household:
34 bulbs (estimated, UK households)
Source: Is Switching to LED Bulbs Worth It? - Evergreen Energy

- Average power ratings (W):

- Incandescent: 60W

- Halogen: 45W

- Compact Fluorescent (CFL): 15W

- LED: 8W

Source: Average Wattage of a Light Bulb: A Comprehensive Guide

- Lightbulb type distribution:

  - Incandescent: 5%

  - Halogen: 10%

  - Compact Fluorescent (CFL): 10%

  - LED: 75%

Disclaimer: We could not find any modern data regarding these statistics.

These percentages are estimations guided by the predictions / trends outlined by following sources:

Microsoft Word - Revised Lighting Report 070414b

More on the DOE's 2020 US Lighting Market Characterization - US Lighting Trends

Relating to Household Data (UK)

- Average usable floor space per house:

97m2 (2.11, 2021)

Source: English Housing Survey 2021 to 2022: headline report - GOV.UK

- Average people per household:
2.2 people (2023-2024 English Housing Survey)
Source: Chapter 1: Profile of households and dwellings - GOV.UK

PROGRAM / ALGORITHM

Constants

DAYS_IN_YEARS = 365.25

HOURS_PER_DAY = 3.9

COST_PER_KWH = 0.2703

CO2_PER_KWH = 0.207074

INCANDESCENT_POWER = 60

HALOGEN_POWER = 45

CFL_POWER = 15

LED_POWER = 8

Variables

incandescent_count = int

halogen_count = int

cfl_count = int

led_count = int

room_count = int

people_count = int

For annual light usage estimate for census

Proportioned lightbulb type distribution with 34 lightbulbs

C_INCANDSCENT_COUNT = 2

C_HALOGEN_COUNT = 3

C_CFL_COUNT = 3

C_LED_COUNT = 26

C_PEOPLE_COUNT = 2.4

C_ROOM_COUNT =  5 (Living room, kitchen, 2 bedrooms, bathroom)

Calculation of Annual Energy Consumption

ΣW = (nI)(WI) +(nH)(WH) + (nCF)(WCF) + (nLED)(WLED)

Where n = no. of, W = Power (W) of, I = Incandescent, H = Halogen, CF = Compact Fluorescent, LED = Light Emitting Diode

ENERGYAnnual = (ΣW/1000)(hours used per day)(days in years)(people / no. rooms)

Calculation of Estimated Annual Cost

COSTAnnual = (EAnnual)(Cost per kWh)

Calculation of Estimated Annual CO2e Emissions

CO2Annual = (EAnnual)(CO₂e per kWh)